Written Part:

Q.1

- What is the bit-rate produced by the camera?

Solution:

450  520  25 = 5, 850,000 pixels / second

As mentioned in the question, each sample of Y, Cr, Cb is quantized with 8 bits and the color sub sampling scheme is 4:2:0(as known as 4:1:1)

48 + 8 + 8 = 48bits per 4 pixels

Then 48/4 =12bits/pixel

So, Bit rate is:

12 5,850,000 = 70,200,000bits/second

- Suppose we want to store the video signal on a hard disk, and, in order to save space, re-quantize each chrominance (Cr, Cb) signals with only 6 bits per sample. What is the minimum size of the hard disk required to store 10 minutes of video(3 points)

Solution:  because at this time each chrominance (Cr, Cb) signals with only 6 bits per sample, then

(48 + 6 + 6)/4 = 11 bits/pixel

Then,

450  520 25 = 5, 850,000 pixels / second

So, the minimum size of the hard disk required to store 10 minutes of video

11  5,850,000  60  10 = 3.861 1010bits

Q.2

1)Write down the quantized sequence

Solution: to quantize this sequence by dividing the interval [-4,4] into 32 uniformly distributed levels, the length of each interval of [-4,4] is 0.25,

So, the quantized sequence is

{1.75, 2.25, 2.25, 3.25,3.25,3.25,2.5,2.75,2.75,2.75, 1.5,1.0,1.25,1.25,1.75,2.25,2.25,2.25,2.0,2.25,1.25,0.25, -1.25, -1.25,-1.75,-1.0,-2.25,-1.5,-1.5,-0.75,0.0,1.0}

2) How many bits do you need to transmit it? (1 points)

Solution:

Because there are 32 distributed levels, so we need 5 bits to represent each level. And there are 32 numbers here, so the total number of bits we need is 160.

Analysis Questions for part 1

- Let’s try an experiment where s (scale factor) remains constant and n (number of lines) is allowed to vary.

1)When s = 2.0   aliasing = 0

n = 32

n = 64

n =128

n =256

n = 360

2)  When s =4, aliasing = 0

n = 32

n = 64

n = 128

n = 256

n = 360

3) When s =6, aliasing =0

n = 32

n = 64

n = 128

n = 256

n = 360

Analysis:

From the above pictures, we can find that when s is constant and n varies, aliasing becomes more and more obvious when the number of lines increases.

- Let’s try another experiment, this time keep n (number of lines) constant and varying s (scale factor).

  - when n = 32, aliasing = 0

s = 2

s = 4

s= 6

  - When n = 64, aliasing = 0

s = 2

s = 4

s = 6

3）when n = 128, aliasing = 0

s = 2

s = 4

s = 6

- When n = 360, aliasing

s = 2

s = 4

s = 6

Analysis: From the above pictures, we can find that when n is constant and s varies, aliasing becomes more and more obvious when s is larger.

Analysis Questions for part 2

Let’s try an experiment where s (speed of rotation) remains constant and fps is allowed to vary. Study the value of the os (observed speed of rotation), especially when there is temporal aliasing.

- Can you design a formula relating s, fps and os.

The default fps in the left picture is 30

The formula relating s, fps, and os is:

fps/30 * s = os

Evaluate if your formula works for certain values of s and fps. If s = 10 rotations per second,

- What is the observed speed os for an fps of 25?

25/30 * 10  8.3

- What is the observed speed os for an fps of 16?

16/30 * 10 5.3

- What is the observed speed os for an fps of 10?

10/30 * 10  3.3

- What is the observed speed os for an fps of 8?

8/30 *10  2.7